Access door setup for Bosch 4000

To enable Door via Codepad:

Go to location 13098 and choose how many doors are required.

Default is 0 – All doors disabled

Codepad assigned door:

This option is assigning door or doors to codepads for adding access cards

and cycling door operations. Each codepad and be assigned  door/s.

By default this is set to [15] – No door is assigned

If this is left as [15] – you will not be able to:

- Cycle the door operation

- Add access card users via the codepad

Assign Door 1 to Codepad 1

Location 552 – Enter 1 - to assign Door 1 to Codepad 1

Enter 1234 # 	552 # 	1 *

Assigning Access Card/Tag to User:

- Using the Master Code, you can add/ delete access cards/tags just like user codes and remotes.

- Access card credentials are between [601 – 664] which like the remotes are assigned to users 1 - 64 respectively.

- To add an access card credential:

Enter Master code + 1 # + 601 + # Present card at reader (2 beeps are heard when successful).

Enter 25801#	601# 	Present Card/tag at reader

- To delete access card/tag

Enter Master code + 1 # + 601 + # then *  (2 beeps are heard if successful).

Enter 25801# 	601#	* to delete

As mentioned, there are 32 zones on the 4000, there are an additional 4 zones for the access door control. One for each door module [ B901]

These are programmed to same way a normal panel zone and uses the same structure as the 32 zones on the 4000.

- These zones will show up as zones 129 – 132 on the alarm system

- Program  door zone 129 for entry delay 1 on the Door module 1

1234#        13252#        2 *

Access Door Programming Via Alink Software

Common Door Options

Door Name - Name of Door

Door Enable - Yes or No

Exit request - RTE device

Auto door - Unlock during disarm

Fire unlock - Unlocks during FIRE event. Zone to be programmed as a fire zone-type 13

Strike Time - Unlock time of door

Default is 10 seconds

Failure Mode:

Fail Secure - Door remains locked when B901 module is offline

Fail Safe – Access module releases door when B901 module loses communications with 4000 panel

Global Access Settings:

- This is for selecting which weigand format to use. Currently only 3 options.

- 26 bit, 37 bit No site code and 37 bit with site code

Keypad Assigned Door:

- Assign a codepad to an access door.

Assigning user cards/tags:

Enter User Name, Area User Authority, Site Code and Card Number details if required

Zone Programming

- Here you can program the zone for each Door module.

- Each Door module has one zone available for Door input monitoring

Schedule Settings

- This is where you create schedules/timezones for Doors and User Groups. 2 x schedules will need to be programmed per event eg: Door unlocked during business hours – 1 for unlock time and 1 for lock time

- Function: 	No Schedule - Disabled

Lock Door – Locked during valid Schedule

Unlock Door – Unlocked during valid Schedule

Secure Door – Locks during valid Schedule

Activate User – User allowed access during valid Schedule

Deactivate User – User denied access during valid Schedule

Sunday through Saturday 	Yes/No – enable or disable for each day.

- On Date		Yes/No – enable/disable user to edit schedule via codepad.

- Time			Time of day that the schedule starts or is valid

- Date			Start date for the event schedule to start – day and month only

Schedule holiday option

3 x options:	Ignore Holiday setting: Schedule still valid during holidays programmed

Holiday will do schedule: Will operate schedule if no days set in schedule

Holidays are exception day: Schedule does not operate on holiday programme

User Group And Holiday Settings

- Up to 8 User Groups

- 1 Holiday schedule

Users that are assigned User Groups can be assigned to a schedule to restrict access to the door.

- Select [View in Setting] section of the User Group

- Select the users you want have in that User Group and click OK

User Group And Holiday Settings

- Up to 8 User Groups

- 1 Holiday schedule

The Holiday schedule is also setup in this area.

- Select [View in Setting] section of the Holidays section

- Select the dates on the monthly  calender and select OK when done

Enable Schedule

In the Option section, just enable the schedule number to activate that schedule